ARTICLE G

Traffic Visibility, Loading, Parking and Access

SEC. 10-1-90
TRAFFIC VISIBILITY.

(a) On  a corner lot in all zoning districts, no fence, wall, hedge, planting or structure shall be erected, placed, planted or allowed to grow in such a manner as to obstruct vision between a height of two and one-half (2-1/2) feet and ten (10) feet above the centerline grades of the intersecting streets in the area bounded by the street lines of such corner lots and a line joining the points along said street lines thirty-five (35) feet from the point of intersection.

(b) In the case of arterial streets intersecting with other arterial streets or railways, the corner cutoff distances establishing the triangular vision clearance space shall be increased to fifty (50) feet. 

SEC. 10-1-91
LOADING REQUIREMENTS.

(a) LOADING SPACE REQUIREMENTS.  On every lot on which a business, trade or industrial use is hereafter established, space with access to a public street or alley shall be provided as indicated below for the loading and unloading of vehicles off the public right-of-way:

Use




Floor


Loading Space
Retail, wholesale,



2,000-10,000


1

Warehouse, service,


10,000-20,000


1

Manufacturing, and 


20,000-40,000


2

Industrial establishments


40,000-60,000


3







Each additional 50,000
1

Hotels, offices,



5,000-10,000


1

Hospitals, places of


10,000-50,000


2

Public assembly



50,000-100,000

2







Each additional 25,000
1

Funeral homes



2,500-4,000


1







4,000-6,000


1







Each additional 10,000
1

(b) MULTIPEL OR MIXED USES.  Where a building is devoted to more than one use or for different uses, and where the floor area for each use is below the minimum required for a loading space but the aggregate floor area of such uses is above such a minimum, then off-street loading space shall be provided as if the entire building were devoted to that use in the building for which the most loading spaces are required.

(c) LOCATION.  Required off-street loading spaces shall be located on the same lot with the principal use requiring such space.  No loading space shall be located within thirty-five (35) feet of the nearest point of intersection of two (2) streets or require any vehicle to back into a public street. 

(d) DESIGN STANDARDS.  Each off-street loading space shall have a width of at least twelve (12) feet, a length of at least forty (40) feet, and a vertical clearance of at least fifteen (15) feet.  Dimensions for loading spaces in connection with funeral homes shall be reduced to ten (10) feet in width, twenty-five (25) feet in length, and eight (8) feet in vertical clearance.  All loading berths shall be completely screened from residential properties by building walls, or a uniformly-painted solid fence, wall, or door, or any combination thereof, not less than eight (8) feet in height. 

(e) SURFACING.  All open off-street loading berths shall be improved with a compacted macadam base, not less than seven (7) inches thick, surfaced with not less than two (2) inches of asphalt or treated with some comparable all-weather dustless material.

(f) REPAIR AND SERVICE.  No motor vehicle repair work or service of any kind shall be permitted in conjunction with loading facilities provided in any Residence District.

(g) UTILIZATION.  Space allocated to any off-street loading berth shall not, while so allocated, be used to satisfy the space requirements for any off-street parking facilities or portions thereof. 

(h) CENTRAL LOADING.  Central loading facilities may be substituted for loading berths on the individual zoning lots provided the following conditions are fulfilled:

(1) Each zoning lot served shall have direct access to the Central Loading Area without crossing streets or alleys at grade. 

(2) Total berths provided shall meet the requirements based on the sum of the several types of sues served.  (Areas of types of uses may be totaled before computing number of loading berths.)

(3) No zoning lot served shall be more than three hundred (300) feet removed form the Central Loading Area.

(4) The tunnel or ramp connecting the Central Loading Area with the zoning lot served shall be not less than seven (7) feet in width and have a clearance of not less than seven (7) feet.   

SEC. 10-1-92
PARKING REQUIREMENTS.


All new parking lots and all alterations of existing lots shall be subject to the approval of the Planning Commission.  Requests for said parking lots shall be accompanied with detailed plans on landscaping, parking layout, drainage provisions and driveway locations. In all districts, except those areas which are located within any fire zone as designated on the Official Map, there shall be provided at the time any use or building is erected, enlarged, extended, or increased, off-street parking stalls for all vehicles in accordance with the following:

(a) ACCESS.  Adequate access to a public street shall be provided for each parking space.

(b) DESIGN STANDARDS.  

(1) Each required off-street parking space shall have a stall width of at least nine (9) feet and a stall length of at least eighteen (18) feet.  Such space shall have a vertical clearance of at least six and one-half (6-1/2) feet.  Minimum width of aisles providing access to stalls for one-way traffic shall be as follows:  Aisles shall be not less than twenty-four (24) feet wide for ninety (90°) degree parking, eighteen (18) feet wide for sixty (60°) degree parking, fifteen (15) feet wide for forty-five (45°) degree parking (angle shall be measured between centerline of parking space and centerline of aisle), and twelve (12) feet wide for parallel parking.  For parallel parking the minimum length of the parking space shall be increased to twenty-three (23) feet.  No parking area of more than two (2) spaces shall be designed as to require any vehicle to back into a public street. Any parking area of more than five (5) spaces shall be sufficiently screened in the form of a solid fence or shrubbery to protect adjacent residential uses.  Large expanses of unchanneled parking areas shall be avoided by interior landscaping and safety islands.

(c) LOCATION.  

(1) Location to be on the same lot as the principal use of not over four hundred (400) feet from the principal use.

(2) Off-street parking is permitted in all yards of all districts except in the front yards of single-family and multi-family residence districts, but shall not be closer than five (5) feet to a side lot line, right-of-way line, or rear lot line.

(3) Off-Street parking in the single-family residence and multi-family residence districts is permitted in the front yard in the driveway, even though closer than five (5) feet to a side lot line providing the driveway conforms to the requirements in Section 10-1-93.

(d) SURFACING.  All open off-street parking areas, except parking spaces accessory to a single-family dwelling, shall be surfaced with a dustless all-weather material capable of carrying a wheel load of four thousand (4,000) pounds (normally, a two-inch blacktop on a four-inch base or five inches of Portland cement will meet this requirement).  Any parking area for more than five (5) vehicles shall have the aisles and spaces clearly marked.  

(e) LANDSCAPING.  

(1) Landscaped Area.  All public and private off-street parking areas which serve ten (10) vehicles or more and are created or redesigned and rebuilt subsequent to the adoption of this Code shall be provided with accessory landscape areas totaling not less than ten (10%) percent of the surfaced area.  The minimum size of each landscape area shall not be less than one hundred (100) square feet.

(2) Location.  Location of landscape areas, plant materials, and protection afforded the plantings, including curbing and provision for maintenance by the property owner shall be subject to approval by the Planning Commission.

(3) Plans.  All plans for such proposed parking areas, at the discretion of the Planning Commission, shall include a topographic survey or grading plan which shows existing and proposed grades and location of improvements.  The preservation of existing trees, shrubs, and other natural vegetation in the parking area may be included in the calculation of the required minimum landscape area.

(4) Special residential requirements.  Those parking areas for ten (10) or more vehicles if adjoining a residential use shall be screened from such use by a solid wall, fence, evergreen planting of equivalent visual density, or other effective means, built and maintained at a minimum height of five (5) feet.  Where a solidly constructed decorative fence is provided along the interior lot line, the minimum setback for the parking area shall be five (5) feet from said lot line.  Said fence shall be located a minimum of one (1) foot form the said lot line. 

(5) Street Setback Area.  No parking shall be permitted between the street right-of-way line and the building setback line prevailing in the zone in which the proposed parking area is to be located.  The resulting open area shall be planted in grass or otherwise landscaped to create a permanent green area. 

(6) Repair and Service.  No motor vehicle repair work or service of any kind shall be permitted in association with parking facilities provided in Residence Districts.

(7) Lighting.  Any lighting used to illuminate off-street parking areas shall be directed away from residential properties and public streets in such a way as not to create a nuisance.  However, in no case shall such lighting exceed three (3) foot candles measured at the lot line. 

Hotels, motels





1 stall for each guest room plus









1 stall for each 3 employees

Sororities, dormitories, rooming and


1 stall for each bed 

Boarding houses

Retirement homes, orphanages, convents

1 stall for each 4 beds plus

And monasteries    




1 stall for each 2 employees

Hospital, sanitariums, institutions,


1 stall for each 3 beds plus

Rest and nursing homes



1 stall for each 2 employees

Medical and dental clinics



3 stalls for each doctor

Churches, theaters, auditoriums,


1 stall for each 6 seats

Community centers, vocational and night

Schools, and other places of public assembly

Colleges, secondary and elementary schools

1 stall for each 2 employees plus









1 stall for each 5 students of 

       






16 years of age or more

Restaurants, bars, clubs and lodges,


1 stall for each 3 seats and

Places of entertainment



1 space for each 2 employees

Mobile home parks




1 stall for each trailer lot plus

1 stall for every 3 lots for guest parking

Manufacturing and processing plants


1 stall for every 1.3 employees;

(including meat and food processing),

number of employees shall be 

laboratories, and warehouses



construed to mean the maximum








    number on the premises at one time.

Financial institutions, business


1 stall for each 250 square feet

Government and professional offices,

of floor area and 1 stall for each

Retail and service establishments


2 employees

(f) CURBS.  Curbs or barriers shall be installed a minimum of four (4) feet from a property line so as to prevent the parked vehicles from extending over any lot lines.  

(g) NUMBER OF STALLS.  Number of parking stalls required are shown in the following table, except that uses, as itemized in the B-2 Zoning Districts, shall provide eight (8) parking spaces for each one thousand (1,000) square feet of gross floor area.

Use






Minimum Parking Required
Dwellings: Single-family, two-family 


2 stalls for each dwelling unit

and mobile homes

Dwellings: Multi-family




2 stalls for each dwelling unit

Housing for the elderly (non-nursing home)
0.75 space for each dwelling with one-half of these spaces to be built before occupancy and the balance of which spaces shall be reserved until such time as the Common Council may order them installed

Hotels, motels
1 stall for each guest room plus


1 stall for each 3 employees

Sororities, dormitories, rooming and
1 stall for each bed 

Boarding houses

Retirement homes, orphanages, convents
1 stall foe each 4 beds plus 

And monasteries
1 stall for each 2 employees

Hospital, sanitariums, institutions,
1 stall foe each 3 beds plus

Rest and nursing homes
1 stall foe each 2 employees

Medical and dental clinics
3 stalls for each doctor

Churches, theaters, auditoriums, 
1 stall for each 6 seats

Community centers, vocational and night schools,

And other places of public assembly

Colleges, secondary and elementary schools
1 stall foe each 2 employees plus


1 stall foe each 5 students of 16 years of age or more

Restaurants, bars, clubs and lodges, 
1 stall foe each 3 seats and

Places of entertainment
1 space for each 2 employees

Mobile home parks
1 stall foe each trailer lot plus


1 stall foe every 3 lots for guest parking

Manufacturing and processing plants
1 stall for every 1/3 employees;

(including meat and food processing),
number of employees shall be 

laboratories, and warehouses
construed to men the maximum number on the premises at one time.

Financial institutions, business 
1 stall for each 250 square feet

Government and professional offices,
of floor are and 1 stall for each

Retail and service establishments
2 employees

Motor vehicle sales
1 space for each 500 square feet

(new and used)
of floor area used plus one space for each 300 square feet of outdoor display area for each motor vehicle to be displayed.  (This requirement does not include service garages – see above)

Repair shops, retail and service stores
1 space for each 150 square feet of net floor space

Automobile repair garages
1 space for each employee

And service stations
plus 1 space for each 250 square feet of floor area used for repair work 

Bowling alleys
5 spaces for each alley

(h) USES NOT LISTED.  In the case of structures or uses not mentioned, the provision for a use which is similar shall apply.  Floor space or area shall mean the gross floor area inside the exterior walls, where floor space is indicated above as a basis for determining the amount of off-street parking required.

(i) COMBINED USES.  Combinations of any of the above uses shall provide the total of the number of stalls required for each individual use.  Two or more uses may provide required off-street parking spaces in a common parking facility less than the sum of the spaces required for each use individually provided such uses are not operated during the same hours.  A written agreement satisfactory to the City Attorney shall accompany any joint use arrangement

(j) HANDICAPPED PARKING REQUIREMENTS.  In addition to any other requirements relating to parking spaces contained in these ordinances, the provisions contained in Sections 101.13, 346.503, and 346.56, Wis. Stats., and any Wisconsin Administrative Code sections adopted pursuant thereto, are hereby adopted by reference and made applicable to all parking facilities whenever constructed.

(k) CHANGES IN BUILDINGS OR USE.  Whenever a building or use is changed, structurally altered, or enlarged to create a need for an increase of twenty-five (25%) percent or more in the number of existing parking spaces, such spaces shall be provided on the basis of the enlargement or change.  Whenever a building or use is enlarged to the extent of fifty (50%) percent or more in the floor area, said building or sue shall then comply with the parking requirements set forth in the district in which it is located. 

(l) OFF-LOT PARKING.

(1) Required off-street parking spaces shall be located on the same lot with the principal use, or when this requirement cannot be met such parking spaces may be located off-lot provided the parking spaces are located in the same district.  Off-lot parking spaces shall also be held in fee simple ownership by the owner of the use requiring such parking or be leased or rented through a written agreement satisfactory to the City Attorney.

(2) Off-lot parking spaces for residential uses shall be with two hundred fifty (250) feet of the principal entrance or the entrance for the individual occupants for whom the spaces are reserved while the farthest portions of a parking lot for all other uses shall be within three hundred (300) feet of the entrance of the establishment.

(3) Accessory parking may be located in residential districts provided that said lot or property are immediately adjacent to a commercial, business or industrial zoning district.

(4) All off-street parking lots adjoining lots zoned for residential use shall have a minimum setback of ten (10) feet from any interior lot line, except if the adjoining lot is used for legally conforming parking purposes.     

SEC. 10-1-93
DRIVEWAYS.


All driveways installed, altered, changed, replaced, or extended after the effective date of this Chapter shall meet the following requirements and must be approved as to location by the Zoning Administrator:

(a) Island between driveway openings shall be provided with a minimum of six (6) feet between all driveways.

(b) The maximum number of driveway openings for vehicular ingress and egress permitted for lots with a  width less than one hundred (100) feet shall be one (1) and for lots with a width greater than one hundred (100) feet shall be two (2).

(c) Vehicular entrances and exits to drive-in theaters, banks, and restaurants; motels; funeral homes; vehicular sale, service, washing and repair stations; garages, or public parking lots shall be not less than two hundred (200) feet from any pedestrian entrance or exit to a school, college, university, church, hospital, park playground, library, public emergency shelter, or other place of public assembly.

(d) Openings for vehicular ingress and egress shall not exceed thirty (30) feet at the property line and thirty-five 935) feet at the roadway for all uses except the maximum curb opening for all residential districts shall be twenty-five (25) feet at the roadway. 

(e) Driveways shall be at least ten (10) feet wide for one and two-family dwellings, at least eighteen (18) feet for farmsteads, and a maximum of thirty-five (35) feet at the roadway for all other uses except the maximum curb opening for all residential districts shall be twenty-five (25) feet. 

SEC. 10-1-94
HIGHWAY ACCESS.

(a) HIGHWAY ACCESS.  No direct private access shall be permitted to the existing or proposed rights-of-way of expressways, or to any controlled access arterial street without permission of the highway agency that has access control jurisdiction.  No direct public or private access shall be permitted to the existing or proposed rights-of-way of freeways, interstate highways and their interchanges or turning lanes nor to intersecting or interchanging streets within 1,500 feet of the most remote end of the taper of the turning lanes (such as exit and entrance ramps).  No driveway openings shall be permitted within one hundred (100) feet of the intersection of an arterial street right-of-way line.

(b) Access barriers, such as curbing, fencing, ditching, landscaping, or other topographic barriers, shall be erected to prevent unauthorized vehicular ingress or egress to the above specified streets or highways.

(c) Temporary access to the above rights-of-way may be granted by the Zoning Administrator after review and recommendation by the highway agencies having jurisdiction.  Such access permit shall be temporary, revocable, and subject to any conditions required and shall be issued for a period not to exceed twelve (12) months.

SEC. 10-1-95
STORAGE AND PARKING OF RECREATIONAL VEHICLES.

(a) DEFINITIONS – RECREATIONAL VEHICLES.  For purposes of this Section, the following definitions shall apply:

(1) Mobile Home.  Mobile home means a structure, transportable in one or more sections, which is eight (8) body feet or more in width and is thirty-tow (32) body feet or more in length, and which is built on a permanent chassis, and designed to be used as a dwelling with or without permanent foundation, when connected to the required utilities, and includes the plumbing, heating, air-conditioning and electrical systems contained therein.  Length of a mobile home means the distance from the exterior of the front wall (nearest to the exterior of the drawbar and coupling mechanism) to the exterior of the rear wall (at the opposite end of the home) where such walls enclose living or other interior space and such distance includes expandable rooms but not bay windows, porches, drawbars, couplings, hitches, wall and roof extensions, or other attachments.  Width of a mobile home means the distance from the exterior of one side wall to the exterior of the opposite side wall where such walls enclose living or other interior space and such distance includes expandable rooms but not bay windows, porches, wall and roof extensions, or other attachments.

(2) Recreational Vehicle.  Recreational vehicle means any of the following:

a. Travel Trailer means a vehicular, portable structure built on a chassis and on wheels; that is between ten (10) and thirty-six (36) feet long, including the hitch and eight (8) feet or less in width; designated to be used as a temporary dwelling for travel, recreation, vacation or other uses and towed by a car, station wagon or truck.  It includes so-called fifth-wheel units.

b. Pick-up Coach means a structure designated to be mounted on a truck chassis for use as a temporary dwelling for travel, recreation, vacation or other uses.

c. Motor Home means a portable, temporary dwelling to be used for travel, recreation, vacation, or other uses, constructed as an integral part of a self-propelled vehicle.

d. Camping Trailer means a canvas or folding structure mounted on wheels and designed for travel, recreation, vacation or other uses. 
e. Chassis Mounts, Motor Homes and Mini-Motor Homes means recreational structures constructed integrally with a truck or motor-van chassis and incapable of being separated therefrom.
f. Converted and Chopped Vans means recreational structures which are created by altering or changing an existing auto van to make it a recreational vehicle.
g. Boat or Snowmobile Trailer means a vehicle on which a boat or snowmobile may be transported and is towable by a motor vehicle.  When removed from the trailer, boat or snowmobile, for purposes of this Article is termed an unmounted boat or snowmobile.
(3) Boat.  Boat means every description of watercraft used or capable of being used as a means of transportation on water.
(4) Yard, Front, means that part of a lot between the front lot line and front(s) of the principal building on the lot, and extended to both side lot lines.
(5) Yard, Rear, means that part of a lot between the rear lot line and the back(s) of the principal building on the lot, and extended to both side lot lines.
(6) Yard, Side, means that part of a lot not surrounded by building and not in the front or rear yard.
(b) PERMITTED PARKING OR STORAGE OF RECREATIONAL VEHICLES.  In all residential and commercial districts provided for in this Zoning Code, it is permissible to park or store a recreational vehicle or boat and boat trailer on private property in the following manner:
(1) Parking is permitted inside any enclosed structure, which structure otherwise conforms to the zoning requirements of the particular zoning district where located.
(2) Parking is permitted outside in the side yard or rear yard provided it is not nearer than five (5) feet to the lot line.
(3) Parking is permitted outside on a hard surfaced or well-drained gravel driveway, provided:
a. 1.  Space is not available in the rear yard or side yard, or there is no    reasonable access to either the side yard or rear yard.
2.  A corner lot is always deemed to have reasonable access to the rear yard.

3.  A fence is not necessarily deemed to prevent reasonable access.

b. Inside parking is not possible.

c. The unit is parked perpendicular to the front curb.

(4) The body of the recreational vehicle or boat must be at least fifteen (15) feet from the face of any curb.

(5) No part of the unit may extend over the public sidewalk or public right-of-way.

(6) Parking is permitted only for storage purposes.  Recreational vehicles or boats shall not be:

a. Used for dwelling purposes, except for overnight sleeping for a maximum of fourteen (14) days in any one calendar year.  Cooking is not permitted at any time.

b. Permanently connected to sewer lines, water liens, or electricity.  The recreational vehicle may be connected to electricity temporarily for charging batteries and other purposes.

c. Used for storage of goods, materials, or equipment, other than those items considered to be part of the unit or essential for its immediate use.

(7) Notwithstanding the above, a unit may be parked anywhere on the premises during active loading or unloading, and the use of electricity or propane fuel is permitted when necessary to prepare a recreational vehicle for use.

(8) The recreational vehicle or boat shall be owned by the resident on whose property the unit is parked for storage.  

State Law Reference:  Section 30.50, Wis. Stats. 

SEC. 10-1-96
STORAGE OF TRACTORS AND ROAD MACHINERY.


No person, firm or corporation shall park, keep or maintain on properties zoned as residential or multiple residential dwellings, the following types of vehicles: tractors, tractor trailers, semi-trailers, farm tractors in excess of six (6) feet in width, dump trucks, auto wreckers and road machinery.  Said vehicles may not be kept or parked on said premises whether or not they are in enclosed buildings, except for the purposes of unloading or servicing the premises.  

SEC. 10-1-97 THROUGH SEC. 10-1-99
RESERVED FOR FUTURE USE.

