CHAPTER 4

Subdivisions

§ 10-4-1
Introduction and Purpose

§ 10-4-2
Definitions

§ 10-4-3
General Provisions

§ 10-4-4
Procedure for Submitting Subdivisions

§ 10-4-5
Design Standards; Streets and Lots


§ 10-4-6
Design Standards; Block Design

§ 10-4-7
Improvements

§ 10-4-8
Easements

§ 10-4-9
Public Sites and Open Spaces

§ 10-4-10
Construction and Erosion Control – Flora Protection

§ 10-4-11
Variations and Exceptions

§ 10-4-12
Enforcement, Penalties and Remedies

SEC. 10-4-1
INTRODUCTION AND PURPOSE.
(a)
In accordance with the authority granted by section 236.45 of the Wisconsin Statutes and for the purposes listed in section 236.01 and 236.45 of the Wisconsin Statutes, the Common Council of the City of Loyal does hereby ordain as follows:

(1)
The provisions of this Chapter shall be held to be minimum requirements adopted to promote the health, safety, morals, comfort, prosperity and general welfare of the City.

(2)
This Chapter shall not repeal, impair or modify private covenants or public ordinances, except that it shall apply whenever it imposes stricter restrictions on land use.

SEC. 10-4-2
DEFINITIONS.
(a)
ALLEY.  A public right-of-way usually not less than thirty (30) feet in width which normally affords a secondary means of vehicular access to abutting property.

(b)
ARTERIAL STREET.  A street which provides for the movement of relatively heavy traffic to, from, or within the City.  It has a secondary function of providing access to abutting land.   An arterial street system may be designated on the City’s master plan.

(c)
COLLECTOR STREET.  A street which collects and distributes internal traffic within an urban area such as a residential neighborhood, between arterial and local streets.  It provides access to abutting property.

(d)
CUL-DE-SAC.  A short street having but one end open to traffic and the other end being permanently terminated in a vehicular turnaround.

(e)
LOCAL STREET.  A street of little or no continuity designed to provide access to abutting property and leading into collector streets.

(f)
LOT.  A piece, parcel or plot of land intended for building development or as a unit for transfer of ownership.

(g)
PLAT.  The map, drawing or chart on which the sub-divider’s plan of subdivision is presented to the Planning Commission for its recommendation to the Common Council for approval.

(h)
SUBDIVISION.  Any division of a parcel of land by the owner or his agent for the purpose of sale or building development where:

(1)
The act of division creates two or more parcels of two (2) acres or less each. 

(2)
Two or more parcels of two (2) acres or less each are created by successive divisions within a five-year period.

(3)
Provided, however, that this Chapter shall not apply to:

a.
Transfers of interests in land by will or pursuant to court order;

b.
Leases for a term not to exceed ten years, mortgages or easements;

c.
The sale or exchange of parcels of land between owners of adjoining property if additional lots are not thereby created and the lots resulting are not reduced below the minimum sizes required by this Chapter or other applicable laws or ordinances. 

SEC. 10-4-3
GENERAL PROVISIONS.
(a)
No division of land within the City or within its extra-territorial plat approval jurisdiction shall be permitted if it results in a subdivision unless a plat of the subdivisions is permitted and approved in accordance with this Chapter and Chapter 236 of the Wisconsin Statutes.

(b)
In addition to the provisions of this Chapter and Chapter 236 of the Wisconsin Statutes, all subdivision shall conform to the Zoning requirements and the Comprehensive Master Plan for the City of Loyal and its extra-territorial planning area, or any future such zoning requirements or Master Plan when adopted by the Common Council.  

SEC. 10-4-4
PROCEDURE FOR SUBMITTING SUBDIVISINS.
(a)
SUBDIVISIONS.

(1)
Preliminary Meetings.  Before filing a preliminary plat, the sub-divider is encouraged to consult with the Planning Commission and/or its consulting staff for advice regarding general requirements affecting the proposed development.  A sketch of the proposed subdivision drawn on a topographic survey map should be submitted.  The sub-divider shall also submit a location map showing the relationship of the proposed subdivision to traffic arteries and existing community facilities.

(2)
Preliminary Plat.

a.
The sub-divider shall submit to the Common Council and to those agencies having the authority to object to plats under provisions in Chapter 236 of the Wisconsin Statutes, a preliminary plat based upon an accurate exterior boundary survey by a registered land surveyor which shall show clearly the proposed subdivision at a scale of not more than one inch per 100 feet having two-foot contour intervals, shall identify the improvements (grading, tree planting, paving, installation of facilities and dedications of land), easements which the sub-divider proposes to make and shall indicate by accompanying letter when the improvements will be provided.  Any proposed restrictive covenants for the land involved shall be submitted.  

b.
The Planning Commission and Common Council may submit a copy of the preliminary plat to the City Engineer and Land Planning Consultant for review and written report of their reaction to the proposed plat.

c.
After review of the preliminary plat and negotiations with the sub-divider on changes being advisable and the kind and extent of public improvements which will be required, the Common Council shall reject or approve conditionally the preliminary plat within forty (40) days, as provided by statute.  The sub-divider shall be informed of any conditions of approval or the reasons for rejection.  Failure of the Common Council to act within forty (40) days shall constitute approval of the preliminary plat unless other authorized agencies object to the plat.

d.
Approval o the preliminary plat shall entitle the sub-divider to final approval of the layout shown by such plat, provide the final plat conforms substantially to such layout and conditions of approval have been met. 

(3)
Final Plat.

a.
Final Plats shall be submitted to the Planning Commission and Common Council within six (6) months of preliminary plat acceptance unless this requirement is waived in writing by the Common Council.

b.
The final plat shall conform to the preliminary plat as approved and to the requirements of all applicable ordinances and state laws and shall be submitted for certification of those agencies having the authority to object to the plat as provided by section 236.12(2), Wisconsin Statutes.

c.
The final plat shall be accompanied by detailed construction plans of all improvements.

d.
The final plat shall be presented to the Common Council at least ten (10) work days prior to the meeting at which it is to be considered and shall be accepted or rejected by the Common Council within sixty (60) days of its submission, unless the time is extended by an agreement with the sub-divider.  Reasons for rejection shall be stated in the minutes of the Common Council meeting and a copy thereof or a written statement of such reasons supplied to the sub-divider.  If the Common Council fails to act within sixty (60) days and the time has not been extended by agreement and if no unsatisfied objections have been filed within that period, the plat as certified by all necessary officials, the plat shall be recorded in accordance with the requirements of the Wisconsin Statutes.  The plat shall be recorded prior to the time that lots are offered for sale, reference is made to the map for sales purposes, or use is made of lot and block numbers on the plat.

e.
If the original of the final plat has been filed with another approving authority, the subdivide may file a true copy of such plat in lieu of the original.  However, before approval of the Planning Commission and Common Council will be inscribed on the original of the final plat, the surveyor or sub-divider shall certify the respect in which the original of the final plat differs from the true copy and all modifications must first be approved.

(b)
MINOR SUBDIVISIONS.

(1)
Certified Survey Required.  When it is proposed to divide land into two parcels or building sites, any one of which is less than five acres, or when it is proposed to divide a block, lot or outlot into not more than two parcels or building sites within a recorded subdivision plat without changing the exterior boundaries of the block, lot, or outlot, the sub-divider shall subdivide by use of a certified survey map, prepared in accordance with section 236.34, Wisconsin Statutes. 

(2)
Procedure.

a.
The sub-divider shall first consult with the Planning Commission regarding the requirements for minor sub-divisional certified surveys before submission of the final map.  Following consultation, a copy of the final map in the form of a certified survey map shall be submitted to the Common Council at least ten (10) days prior to the regular meeting of the Common Council.

b.
The Common Council shall, within forty (40) days of the filing, approved conditionally or reject the map and shall notify the sub-divider of its decision.

c.
The sub-divider shall record the map with the applicable county register of deeds within thirty (30) days of its approval by the Common Council and any other approving agencies.  Failure to do so shall necessitate re-approval of the map by the Common Council.

(3)
Requirements.  To the extent reasonably practicable, the certified survey/minor subdivision plat shall comply with the provisions of this Chapter relating to general requirements, design standards and required improvements.

(c)
FEES.

(1)
General Provisions Regarding Fees. The sub-divider shall pay the City all fees as hereinafter required before being entitled to record a plat or certified survey map. The Common Council may waive all or a portion of the fee requirements for subdivisions outside the City limits, but within the extraterritorial plat review jurisdiction.

(2)
Preliminary Plat and Certified Survey Map Review Fee. The sub-divider shall pay a fee of fifteen ($15.00) Dollars plus Two ($2.00) Dollars for each lot or parcel within the preliminary plat or certified survey may to the City Clerk-Treasurer at the time of the first application for approval of any preliminary plat or certified survey may to assist in defraying the cost of review.  A reapplication fee of Five ($5.00) Dollars shall be paid to the City Clerk-Treasurer at the time reapplication for approval of any preliminary plat or certified survey map which has previously been reviewed. 

(3)
Final Plat Review Fee. The sub-divider shall pay a fee amounting to Five ($5.00) Dollars plus fifty ($.50) cents for each lot or parcel within the final plat to the City Clerk-Treasurer at the time of the first application for final plat approval of the time of the application for final plat approval of said plat to assist in defraying the cost of review.  A reapplication fee amounting to Five ($5.00) Dollars shall be paid to the City Clerk-Treasurer at the time of the reapplication for approval of any final play which has previously been reviewed.

SEC. 10-4-5
DESIGN STANDARDS; STREETS AND LOTS.
(a) STREET DEDICATION.  The sub-divider shall dedicate land and improve streets as provided herein.  Streets shall be located with due regard to topographical conditions, natural fixtures, existing and proposed streets, utilities and land uses and public convenience and safety.

(b) FRONTAGE. All lots shall have sufficient frontage on a public street to allow access by emergency and service motor vehicles.

(c) LOCATION. Street locations shall be consistent with any street plans officially adopted by the City.  All street right-of-way widths, radii or curvature and grades shall conform to the following requirements:

Right-of-Way

Right-of-Way

Pavement Width




Width to be 

Width to be

(face of curb to 

Street Type

  Reserved

  Dedicated

  face of curb)


Arterial Streets

4-Lane Divided
120 feet

100 feet

Dual: 34 feet











(20-foot median)

Not Dived

70 feet


70 feet


48 feet

Collector Streets
66 feet


66 feet


44 feet

Minor Streets, 1,000
60 feet


60 feet


34 feet

Feet or more in length

Minor Streets less
50 feet


50 feet


32 feet

Than 1,000 feet in

Length, Cul-de-sac and 

Frontage Streets

Alleys


25 feet


25 feet


20 feet

(d) CONTINUATION. Streets shall be laid out to provide for possible continuation wherever topographic and other physical conditions permit.  Curbs shall be of equal elevation on each side of a street and shall be crowned. 

(e) MINOR STREET DESIGN.

(1) Minor streets shall be so laid out so as to discourage their use by through traffic

(2) The number of practical intersections of minor streets with major streets shall be reduced to the practical minimum consistent with circulation needs and safety requirements.

(f)
GRADES

(1)
Unless necessitated by exceptional topography subject to the approval of the 


Planning Commission the maximum centerline grade of any street or public way 


shall not exceed the following

a.
Arterial Street: six (6%) percent.

b.
Collector Streets: eight (8%) percent.

c.
Minor streets, alleys and frontage streets: ten (10%) percent.

d.
Pedestrian ways:  twelve (12%) percent unless steps of acceptable design are 


    provided.

e.
The grade of any street shall in case exceed (12%) percent or be less than one-


    half of one (0.5%) percent.

(2) Street grades shall be established wherever practicable so as to avid excessive grading, promiscuous removal of ground cover and tree growth, and the general leveling of the topography. 

(g) FRONTAGE ROADS. Where a subdivision abuts or contains an existing or proposed arterial highway, the Common Council may require a frontage road, non-access reservation along the rear of the property contiguous to such highway, or such other treatment as may be necessary to ensure safe, efficient traffic flow and adequate protection of residential properties. 

(h) RESERVE STRIPS. Reserve strips controlling access to streets shall be prohibited.

(i) TANGENTS. A tangent at least one hundred (100) feet long shall be required between reverse curves on arterial and collector streets.

(j) INTERSECTION VISIBILITY. Streets shall afford maximum visibility and safety and shall intersect at right angles, where practicable.

(k) HALF-STREETS.  Where, on the date of enactment of this Chapter, an existing dedicated or platted half-street is adjacent to the tract being subdivided, the other half of the street shall be dedicated by the sub-divider. The platting of new half-streets is prohibited.

(l) CUL-DE-SAC OR DEAD-END STREETS. Streets designed to have one end permanently closed shall not exceed five hundred (500) feet in length and shall terminate with a turnaround of not less than one hundred (100) feet in diameter of a right-of-way and roadway turnaround of eighty (80) in diameter. 

(m) LOT LINE DESIGN. Where possible, lot lines shall be perpendicular to straight street lines and radial to curved street lines.

(n) LOTS RESTRICTED TO MUNICIPAL BOUNDARIES. Lots shall follow, rather than cross, municipal boundary lines whenever practicable.

(o) STREET NAMES. No street names shall be used which will duplicate or may be confused with the names of existing streets.  Streets names shall be subject to the approval of the Common Council.

(p) LOT SIZES.  Lots shall conform to the size and frontage requirements for the zoning district in which they are located. 

(q) HARDSHIP TO OWNERS OF ADJOINING PROPERTY. The street arrangements shall not be such as to cause hardship to owners or adjoining property in platting their own land and providing convenient access to it. 

(r) ALLEYS.  Alleys shall be prohibited in residential areas unless special permission is granted by the Common Council for their provision.  Alley intersections and sharp changes in alignment shall be avoided, but where necessary, corners shall be cut off sufficiently to permit vehicular movement.  Dead-end alleys shall be avoided, but if unavoidable, shall be provided with adequate turnaround facilities at the dead end.

(s)  MAINTENANCE.  Streets included in approved subdivisions, except designated State, Federal or County roads, shall be maintained by the City in which said streets are located unless other written arrangements are made prior to final plat approval.

(t) LOT REMNANTS. Lot remnants which are below the minimum lot size must be added t adjacent or surrounding lots rather than be allowed to remain as an unusual outlot or parcel unless the owner can show plans for the future use of such remnants.

(u) VERTICAL CURVES.  All changes in street grades shall be connected by vertical curves of a minimum length equivalent to fifteen (15) times the algebraic difference in the rate of grade for arterial streets and collector streets and one-half (1/2) of this minimum length for all other streets.

(v) TANGENTS.  A tangent at least one hundred (100) feet long shall be introduced between reverse curves on arterial and collector streets.

SEC. 10-4-6
DESIGN STANDARDS; BLOCK DESIGN.
(a) The lengths, widths and shapes of blocks shall be appropriate for the topography and the type of development contemplated.

(b) Wherever possible, right angle street intersections should be encouraged.

(c) To provide adequate access and circulation to community facilities the Common Council may require that sidewalks be provided, either along streets through the center of blocks.  Center crosswalks shall not be less than ten (10) feet wide. 

(d) The Common Council may require that certain species of trees be planted on both sides of all streets. 

SEC. 10-4-7
IMPROVEMENTS.
(a) GRADING. After the installation of temporary block corner monuments by the sub-divider and establishment of street grades by the Common Council, the Common Council shall cause to be graded the full width of the right-of-way of all streets to be dedicated in accordance with plans and standard specifications approved by the Common Council. 

(b) SURFACING. After the installation of all utility improvements, the City, at its own discretion, shall surface all roadways in streets dedicated to the widths prescribed by regulations of the City. 

(c) CURB AND GUTTER. After the installation of all utility improvements, the City, at its own discretion, shall construct concrete curbs and gutters in accordance with plans and standard specifications approved by the Common Council.  The sub-divider or property owner, at the time of curb and gutter installation shall be assess for fifty percent (50%) of the cost of curb and gutter, engineering and excavation. 

(d) SIDEWALKS.

(1) The Common Council may require the sub-divider to construct a concrete   sidewalk on one side of all frontage streets and both sides of all other streets within the subdivision.  The Common Council may permit the construction of a concrete sidewalk on only one side of minor streets that serve lots having an average width of one hundred (100) feet or more fronting on said street and may waive the construction of sidewalks on collector and minor streets that serve lots having an average width of one hundred fifty (150) feet or more fronting on said street.  The construction of all sidewalks shall be in accordance with plans and standard specifications approved by the Common Council.

(2)
Wider than standard sidewalks may be required by the Common Council in the vicinity of schools, commercial areas and other places or public assemblage; and the Common Council may require the construction of sidewalks in locations other than required under the preceding provisions of this Chapter if such walks are necessary, in their opinion, for safe and adequate pedestrian circulation. 

(e)
PUBLIC SANITARY SEWERAGE AND PRIVATE SEWAGE DISPOSAL 
SYSTEMS.

(1)
The city, at its own discretion, shall construct sanitary sewers in such a 

      manner as to make adequate sanitary sewerage service available to each lot  within the subdivision.  This installation shall be subject to ordinance SEC 10- 4-7 (j).  If public sewer facilities are not available, the sub-divider shall make provisions for adequate private sewerage disposal systems.  The Common Council may require the installation of sewer laterals to the street lot line.  If, at the time of final platting, sanitary sewer facilities are not available to the plat but will become available within 
a period of five (5) years from the date of plat recording, the sub-divider shall install
or cause to be installed sanitary sewers and sewer laterals to the street lot line in 
accordance with this section and shall cay all laterals as may be specified by the Common Council.  The size, type and installation of all sanitary sewers proposed with plans and standard specifications approved by the Common Council.

(2)
Sub-divider shall assume fifty percent (100%) of the cost of installing all sanitary    sewers eight 98) inches in diameter or less in size.  If greater than eight (8) inch diameter sewers are required to handle the contemplated sewage flows. The cost of such larger sewers shall be prorated with the Sub-divider assuming the cost of installing fifty percent (100%) of what it would cost to install an eight (8) inch in diameter sewer, the excess cost to be borne by the City.

(3)
Holding tanks may be allowed to be installed on real estate owed and used by a tax-exempt public entity only upon showing of hardship to the City Council, and an affirmative majority vote by said City Council to approve of such requested variance.  A determination of hardship shall be made upon such information and documentation as the City Council may request of said tax exempt public entity prior to any vote on said proposed variance by the City Council.

(f)
PUBLIC WATER SUPPLY FACILITIES.

(1)
The City, at its own discretion, shall construct water mains in such a manner as to make adequate water service available to each lot within the subdivision.  If municipal water is not available, the sub-divider shall make provision for adequate private water systems as specified by the City and other applicable State and County regulations.  The Common Council may require the installation of water laterals to the street lot line.  The size, type and installation of all public water mains proposed to be constructed shall be in accordance with plans and standard specifications approved by the City Engineer.

(2)
The sub-divider shall assume the cost of installing all water mains then (10) inches in diameter or less in size.  If greater than then (10) inch diameter mains are required, the excess cost of such mains over and above the cost of a then (10) inch main shall be borne by the City.

(g)
OTHER UTILITIES.

(1)
The sub-divider shall cause gas, electrical power and telephone facilities to be                                   installed in such a manner as to make adequate service available to each lot in the subdivision.  No such electrical or telephone service shall be located on overhead poles along the front lot lines unless otherwise allowed due to exceptional topography or other physical barrier.

(2)
Plans indicating the proposed location of all gas, electrical power and telephone distribution and transmission lines required to serve the plat shall be approved by the City Engineer.

(h)
STREET LAMPS AND STREET SIGNS.

The City shall cause to be installed street lamps and street signs, where deemed necessary by the sub-divider and Common Council.

(i)
SUBDIVISION DEVELOPMENT.

A minimum of two thirds (2/3) al all lots on one street shall have buildings erected and sold, before the Common Council will participate in the development, beyond initial grading, of any other streets, in that subdivision.

(j)
SPECIFICATION; PLANS.  Unless otherwise stated, all of the required improvements shall conform to engineering standards and specifications as required by the Common Council.  Such improvements shall be made in sequence as determined by the City Engineer.  The following plans and accompanying construction specifications may be required by the Common Council before authorization of construction or installation of improvements:  (1)  Street plans and profiles showing existing and proposed grades, elevations    and cross-sections of required improvements.     

(1)
Sanitary sewer plans and profiles showing the locations, grades, sizes, elevations and materials of required facilities.

(2)
Storm sewer plans and profiles showing the locations, grades, sizes, cross-sections, elevations and materials of required facilities.

(3)
Water main plans and profiles showing the locations, sizes, elevations and materials of required facilities.

(4)
Erosion and sedimentation control plans showing those structures required to retard the rate of runoff water and those grading excavation practices that will prevent erosion and sedimentation.  Such plans shall generally follow the guidelines and standards set forth in the publications, Minimizing Erosion in Urbanizing Areas, as prepared by the U.S. Department of Agriculture, Soil Conservation Service, 1972.

(5)
Planting plans showing locations, age, caliper and species of any required grasses, vines, shrubs and trees.

(6)
Additional special plans or information, as required.

(k)
FINANCING.  Before a Final Plat is approved by the Common Council, The sub-divider shall submit an agreement and performance bond or cash escrow agreement to assure the following:

(1)
The sub-divider shall pay for the cost of all improvements required in the                subdivision by the Common Council.

(2)
Guaranteed completion of the required improvements within a three-year period.

(3)
Payment by the sub-divider for all costs incurred by the City for review ad inspection.  This would include preparation and review of plans and specifications by the Engineer, Planner and Attorney, as well as other costs of a similar nature.

(4)
The City may elect to install any of the required improvements under the terms of a cash escrow agreement.

(5)
The performance bond or cash escrow agreement shall be equal to one and one-quarter (1-1/4) times the Engineer’s estimated cost of the required improvements.

(6)
If the required improvements are not complete within the three-year period, all amounts held under the escrow agreement or performance bond shall be turned over and delivered to the City and applied to the cost of the required improvements.  Any balance remaining after such improvements have been made shall be returned to the owner or sub-divider.  The Common council at its option, my extend the bond period for an additional period not to exceed three (3) years.                                  

SEC. 10-4-8
EASEMENTS.

(a) UTILITY EASEMENTS.  The Common Council shall require easements for poles, wires, conduits, storm and sanitary sewers, gas, water and head mains or other utility lines to be provided and paid for by the sub-divider at the option of the Common Council.  It is the interest of this Chapter to protect all established easements so as to assure proper grade, assure maintenance of the established grade, prohibit construction of permanent fences or retaining walls over under ground installation and prevent the planting of trees in the easement area.

(b) DRAINAGE EAESMENTS.  Where a subdivision or land division is traversed by a watercourse, drainage way channel or stream, or low area, an adequate drainage way width, alignment and improvement of such drainage way or easement shall be subject to the approval of the Common Council; and parallel streets or parkways may be required in connection therewith.  Where necessary, storm water drainage shall be maintained by landscaped open channels of adequate size and grade to hydraulically accommodate maximum potential volumes of flow.  These design details are subject to review and approval by the Common Council.

SEC. 10-4-9
PUBLIC SITES AND OPEN SPACES.

(a) DEDICATION AND RESERVATION OF LANDS.  Whenever a tract of land to be subdivided embraces all or any part of a street, drainage way or other public way which has been designated in the master plan, comprehensive plan component or on the official map of the City, said public way shall be made part of the plat and dedicated or reserved by the sub-divider in the locations and dimensions indicated on said plan or maps and in accordance with this Section.

(b) DETERMINATION.  Whenever a proposed school site, park, playground, greenway, open space or other public land, other than streets or drainage ways, designated in the master plan, comprehensive plan component or on the official map of the City is embraced, all or in part, in a tract of land to be subdivided, these proposed public lands shall be made a part of the plat and shall be dedicated to the public at the rate of one (1) acre for each twenty-five (25) proposed dwelling units; and said proposed public lands, other than streets or drainage ways, in excess of the rate established herein shall be reserved for a period not to exceed one (1) year from the date of final plat approval unless extended by mutual agreement of purchase by the public agency having jurisdiction, or unless extended by a mutual agreement for acquisition by the City.  The purchase price shall be the present fair market value of said lands in an undeveloped state as indicated by impartial appraisal.  Costs for said appraisal are to be shared equally between developer and City.

(c) PROPERTIONATE PAYMENTS IN LIEU OF DEDICATION.

(1) a.  If the amount of land required to be dedicated, other than for streets and drainage ways as indicated on the master plan, comprehensive plan component or official map of the City totals less than the ratio of one (1) acre for each twenty-five (25) proposed dwelling units, the sub-divider shall pay to the City a fee equivalent to the fair market value of the amount of land representing the difference between the amount of land required to be dedicated, other than the streets and drainage ways, as indicated on said plans or maps and the rate of dedication established herein.

          b. Said fees required in lieu of dedication shall be paid to the City Clerk-Treasurer at the time of first application for approval of a final plat of said subdivision.  

     (2)  Special Fund.  All funds so collected by the City shall be deposited as          “Special Fund for the Acquisition and Development of Public Sites, Recreation Areas, Open Spaces and Greenways” and that said funds so levied and collected shall be used for such purposes at such places and in such manner as shall be approved, ordered and directed by the City upon recommendation by the Planning Commission.  Any and all interest accumulated upon such funds shall be added to the Special Fund and be used only for acquisition and developments for said purposes.

(d)  DEVELOPMENT OF DEDICATED AREA.  It shall be the duty of the City to properly develop and maintain the dedicated areas and the owner who dedicated said land shall in no way be responsible for its development, maintenance or liability thereon except that said owner shall not develop the surrounding area in a manner which would unduly depreciate the purpose, use or value of the dedicated property and except if such owner shall reside on one of the subdivided parcels in which case he shall be responsible for the maintenance of adjacent public property as may be required in other laws of the City. 

SEC. 10-4-10
CONSTRUCTION AND EROSIONCONTROL – FLORA PROTECTION.

(a) EROSION CONTROL.  The sub-divider shall cause all grading, excavations, open cuts, side slopes and other land surface disturbances to be so mulched, seeded, sodded or otherwise protected that erosion, silt ration, sedimentation and washing are prevented in accordance with the plans and specifications approved by the Common Council. 

In addition:

(1) Sod shall be laid in strips at those intervals necessary to prevent erosion and at right angles to the direction of drainage.

(2) Temporary vegetation and mulching shall be used to protect critical areas, and permanent vegetation shall be installed as soon as practical.

(3) Construction at any given time shall be confined to the smallest practical area and for the shortest practical period of time.

(4) Sediment basins shall be installed and maintained at all drainage ways to trap, remove and prevent sediment and debris from being washed outside the area being developed.

(b) PROTECTING EXISTING FLORA.  The subdivide shall make every effort to protect and retain all existing trees, shrubbery, vines and grasses not actually lying in public roadways, drainage ways, building foundation sites, private driveways, soil absorption waste disposal areas, paths and trails.  Such trees are to be protected and preserved during construction in accordance with sound conservation practices, including the preservation of trees by well islands or retaining walls whenever abutting grades are altered.

SEC. 10-4-11
VARIATIONS AND EXCEPTIONS.

 (a)
Where the Common Council Finds that extraordinary hardships or particular difficulties may result from strict compliance with these regulations, it may recommend to the Common council variations or exceptions to the regulations so that substantial justice may be done and the public interest secured, provided that such variation or exception shall not have the effect of nullifying the intent and purpose of this Chapter. 

(b) The Common Council shall not recommend variations or exceptions to the regulations of the Chapter unless it shall make findings based upon the evidence presented to it in each specific case that:

      (1)  The granting of the variation will not be detrimental to the public safety, health, or welfare or injurious to other property or improvements in the neighborhood in which the property is located;

      (2)  the conditions upon which the request for a variation is based are unique to the property for which the variation is sought and are not applicable, generally to other property;

     (3)  Because of the particular physical surroundings, shape or topographical conditions of the specific property involved, a particular hardship to the owner would result, as distinguished from a mere inconvenience, if the strict letter of the regulations were carried out.

SEC. 10-4-12
ENFORCEMENT, PENALTIES AND REMEDIES.

(a) The city Building Inspector and Zoning Officer shall have primary responsibility for enforcing this Chapter.  No building permit shall be issued for construction on any lot until the final plat for the subdivision has been duly recorded, or a certified survey map is recorded.

(b)  Any person, firm or corporation who fails to comply with the provisions of this Chapter shall, upon conviction thereof, forfeit no less than Twenty-Five ($25.00) Dollars nor more than Five Hundred ($500.00) Dollars and the costs of prosecution for each violation and in default of payment of such forfeiture costs shall be imprisoned in the County jail until payment thereof, but not exceeding thirty (30) days.  Each day a violation exists or continues shall constitute a separate offense.  In addition, the remedies provided by sections 236.30 and 236.31 of the Wisconsin Statutes shall be available to the City.
