CHAPTER 2

Sewer Use

Article A
Definitions – Alphabetical
§ 5-2-1

Definitions

Article B
Use of the Public Sewers
§ 5-2-2

Sanitary Sewers

§ 5-2-3

User Connection

§ 5-2-4

Storm Sewers

§ 5-2-5

Prohibitions and Limitations on Sewer Discharge

§ 5-2-6

Special Arrangements

§ 5-2-7

New Connections

§ 5-2-8

Establishment of Service

§ 5-2-9

Reserved for Future Use

Article C
Control of High Strength – Toxic Wastes Directed to Public Sewers
§ 5-2-10
Submission of Basic Data

§ 5-2-11
Extension of Time

§ 5-2-12
High Strength – Toxic Discharges

§ 5-2-13
Control Manholes

§ 5-2-14
Metering of Waste

§ 5-2-15
Waste Sampling

§ 5-2-16
Analyses

§ 5-2-17
Pretreatment

§ 5-2-18
Submission of Information

§ 5-2-19
Grease and/or Sand Interceptors

Article D
Right of Entry, Safety and Identification
§ 5-2-20
Right of Entry

§ 5-2-21
Safety

§ 5-2-22
Identification, Right to Enter Easements

§ 5-2-23
through

§ 5-2-24
Reserved for Future Use

Article E
Sewer Construction (Building Sewers)
§ 5-2-30
Work Authorized

§ 5-2-31
Cost of Sewer Connection

§ 5-2-32
Use of Old Building Sewers

§ 5-2-33
Materials and Methods of Construction

§ 5-2-34
Building Sewer Grade

§ 5-2-35
Storm and Groundwater Drains

§ 5-2-36
Conformance to Plumbing Codes

§ 5-2-37
Inspection of Connection

§ 5-2-38
Barricades; Restoration

§ 5-2-39 
Reserved for Future Use

Article F
Septic Tank and Holding Tank Disposal
§ 5-2-40
Discharge of Holding Tank Wastes Into System

§ 5-2-41
through

§ 5-2-49
Reserved for Future Use

Article G
Damage or Tampering With Facilities
§ 5-2-50
Willful, Negligent or Malicious Damage

§ 5-2-51
Liability to Discharger for Losses

§ 5-2-52
through

§ 5-2-59
Reserved for Future Use

Article H
Violations and Penalties

§ 5-2-60
Written Notice of Violation


§ 5-2-61
Accidental Discharge

§ 5-2-62
Continued Violations

§ 5-2-63
Liability to the City for Losses

§ 5-2-64
through 

§ 5-2-65
Reserved for Future Use

Article I
Appeals

§ 5-2-70
Procedures

§ 5-2-71
through

§ 5-2-79
Reserved for Future Use

Article J
Validity

§ 5-2-80
Repeal of Conflicting Ordinances

§ 5-2-81
Invalidation Clause

§ 5-2-82
Amendment

§ 5-2-83
through 

§ 5-2-89
Reserved for Future Use

Article K
Audit
§ 5-2-90
Annual Audit

§ 5-2-91
through

§ 5-2-99
Reserved for Future Use

Article L
Enacting Clause

§ 5-2-100
Date of Effect

ARTICLE A

Definitions

SEC. 5-2-1
DEFINITIONS.

(a) The following definitions shall be applicable in this Chapter

(1) Ammonia Nitrogen (NH3-N – One of the oxidation state of nitrogen, in which nitrogen is combined with hydrogen in molecular form as NH3 or in ionized form as NH4 +.  Quantitative determination of Ammonia Nitrogen shall be made in accordance with procedures set forth in “Standard Methods.”

(2) Approving Authority shall mean the City Council or its duly authorized deputy, agent, or representative.

(3) Biochemical Oxygen Demand (BOD) shall mean the quantity of oxygen utilized in the biochemical oxidation of organic matter in five (5) days at twenty (20°) degrees Centrigrade, expressed as milligrams per liter.  Quantitative determination of BOD shall be made in accordance with procedures set forth in “Standard Methods.”

(4) Building Drain shall mean that part of the lowest horizontal piping of a drainage system which receives the discharge from soil, waste, and other drainage pipes inside the walls of the building and conveys it to the building sewer, beginning five (5) feet (1.5 meters) outside the inner face of the building walls.

(5) Building Sewer shall mean the extension from the building drain to the public sewer or other place of disposal, also called house connection.  Once constructed, the portion of the building sewer located within the public right of way or easement shall be considered an integral part of the public sewer, and maintenance of this portion shall become the responsibility of the City.

(6) Combined Sewer shall mean any sewer intended to serve as a sanitary sewer and a storm sewer.

(7) Commercial User shall mean any user whose premises are used primarily for ht conduct of a profit-oriented enterprise in the fields of construction, wholesale or retail trade, finance, insurance, real estate or services, and who discharges primarily normal domestic sewage.  This definition shall also include multi-family residences having three or more units served by a single meter. 

(8) Composite Sample – (24 hours) shall be the combination of individual samples taken at intervals of not more than one (1) hour.

(9) Compatible Pollutant means biochemical oxygen demand, suspended solids, pH, or fecal coliform bacteria, plus additional pollutants identified in the NPDES permit for the publicly-owned treatment works receiving the pollutants if such works was designed to treat such additional pollutants, and in part does remove such pollutants to a substantial degree.

(10) Chlorine Requirement shall mean the amount of chlorine, in milligrams per liter, which must be added to sewage to produce a specified residual chlorine content in accordance with procedures set forth in “Standard Methods.”

(11) Easement shall mean an acquired legal right for the specific use of land owned by others.

(12) Floatable Oil is oil, fat, or grease in a physical state such that it will separate by gravity from wastewater by treatment in an approved pretreatment facility.  A wastewater shall be considered free of floatable oil if it is properly pretreated and the wastewater does not interfere with the collection system.

(13) Flow Proportional Sample – A sample taken that is proportional to the volume of flow during the sample period.

(14) Garbage shall mean the residue form the preparation, cooking and dispensing of food, and from the handling, storage, and sale of food products and produce.

(15) Ground Garbage shall mean the residue from the preparation, cooking and dispensing of food that has been shredded to such degree that all particles will be carried freely in suspension under the flow conditions normally prevailing in public sewers with no particle greater than one-half (1/2) inch in any dimension.

(16) Incompatible Pollutant means any pollutant which is not a compatible pollutant.

(17) Industrial User shall mean any user whose premises are not used primarily for the conduct of a profit-oriented enterprise in the fields of manufacturing, transportation, communications or utilities, mining, agriculture, forestry, or fishing.

(18) Industrial Waste – Any trade or process waste as distinct from segregated domestic wastes or wastes from sanitary conveniences.

(19) Institutional User shall mean any user whose premises are used primarily for the conduct of activities of a social, charitable, or educational character.

(20) Intercepting Sewer – A sewer whose primary purpose is to convey sewage from a collection system or systems to a wastewater treatment plant.  Size of the sewer is not a factor.

(21) Natural Outlet shall mean nay outlet, including storm sewers and combined sewer overflows, into a watercourse, pond, ditch, lake, or other body of surface water or groundwaters.

(22) Normal Domestic Sewage shall mean sanitary sewage resulting from the range of normal domestic activities, in which BOD5 and SS concentrations do not exceed normal concentrations of:

a.  A five day, 20°C, BOD of not more than 265 mg/1.

b. A suspended solids content of not more than 305 mg/1.

(23) WPDES Permit – Permit issued under the Wisconsin Pollutant Discharge Elimination System, Chapter NR 210, Wisconsin Administrative Code.
(24) Person shall mean any and all persons including any individual, firm, company, municipal or private corporation, association, society, institution, enterprise, governmental agency, or other entity.
(25) pH shall mean the logarithm of the reciprocal of the hydrogen ions, in grams, per liter of solution.  Neutral water, for example, has a pH value of 7 and a hydrogen-ion concentration of 10-7.
(26) Phosphorus (P) – Total phosphorus in wastewater, which may be present n any of three principal forms: orthophosphate, polyphosphates, and organic phosphates.  Quantitative determination of total phosphorus shall be made in accordance with procedures set forth in “Standard Methods.”
(27) Pretreatment – shall mean an arrangement of devices and structure, for the preliminary treatment or processing of wastewater required to render such wastes acceptable for admission to the public sewers.
(28) Public Authority shall mean any user whose premises are used for the conduct of the legislative, judicial, administrative, or regulatory activities of federal, state, local, or international units of government; government-owned educational facilities; government-owned health facilities; or government-owned recreational facilities.  This does not include government-owned or operated business establishments.
(29) Public Sewer shall mean any sewer provided by or subject to the jurisdiction of boundaries that serve one or more person and ultimately discharge into the City of Loyal sanitary sewer system, even though those sewers may not have been constructed with City of Loyal funds. 
(30) Sanitary Sewage shall mean a combination of water-carried wastes from residences, business buildings, institutions and industrial plants (other than industrial wastes from such plants); together with such ground, surface and storm waters as may be present.
(31) Sanitary Sewer shall mean a sewer that carries sanitary and industrial water- carried wastes from residences, commercial buildings, industrial plants, and institutions, together with minor quantities of ground, storm and surface water that are not admitted intentionally.
(32) Segregated Domestic Wastes may be defined as wastes from nonresidential sources resulting from normal domestic activities.  These activities are distinguished form industrial, trade, and/or process discharge wastes.
(33) Sewage is the spent water of a community.  The preferred term is “wastewater,” Subsection (44).
(34) Sewage System shall meant he composite network of underground conduits carrying wastewater and appurtenances incidental thereto (i.e., manholes, lift stations, service lateral).
(35) Sewer shall mean a pipe or conduit that carries wastewater or drainage water.
(36) “Shall” is mandatory; “May” is permissible.
(37) Slug shall mean any discharge of water or wastewater which in concentration of any given constituent or in quantity of flow exceeds for any period of duration longer than fifteen (15) minutes more than five (5) times the average twenty-four (24) hour concentration of flows during normal operation and shall adversely affect the system and/or performance of the wastewater treatment works.
(38) Standard Methods shall mean the examination and analytical procedures set forth in the most recent edition of “Standard Methods for the Examination of Water, Sewage and Industrial Wastes” published jointly by the American Public Heath Association, and the Federation of Sewage and Industrial Wastes Associations.
(39) Strom Drain (sometimes termed “storm sewer”) shall mean a drain or sewer for conveying water, groundwater, subsurface water, or unpolluted water from any source.
(40) Storm Water Runoff shall mean that portion of the rainfall that is drained into the sewers.
(41) Suspended Solids (SS) shall mean solids that either float on the surface of, or are in suspension in water, wastewater, or other liquid, and that are removable by laboratory filtering as prescribed in “Standard Methods” and are referred to as nonfilterable residue.
(42) Unpolluted Water is water of quality equal to or better than the effluent criteria in effect or water that would not cause violation of receiving water quality standards and would not be benefited by discharge to the sanitary sewers and wastewater treatment facilities provided.
(43) User Classes are categories of users having similar flows and water characteristics; that is, levels of biochemical oxygen demand, suspended solids, phosphorus, ammonia nitrogen, etc.  For the purposes of this ordinance, there shall be four user classes; residential, commercial, industrial and public authority.
(44) Wastewater shall mean the spent water of a community.  From the standpoint of source, it may be a combination of the liquid and water-carried wastes from residences, commercial buildings, industrial plants, and institutions, together with any groundwater, surface water, and storm water that may be present.
(45) Wastewater Facilities shall mean the treatment works defined in Subsection (46) below exclusive of interceptor sewers, and wastewater collection systems.
(46) Wastewater Treatment Works shall mean an arrangement of devices and structures for the storage, treatment, recycling, and reclamation of wastewater, liquid industrial wastes, and sludge.  These systems include interceptor sewers, outfall sewers, wastewater collection systems, individual systems, pumping, power and other equipment and their appurtenances; any works that are an integral part of the treatment process or are used for ultimate disposal or residues from such treatment; or any other method or system for preventing, abating, reducing, storing, treating, separating, or disposing of municipal or industrial wastes. 
(47) Watercourse shall mean a natural or artificial channel for the passage of water either continuously or intermittently. 
ARTICLE B

Use of the Public Sewers

SEC. 5-2-2
SANITARY SEWERS.


No person(s) shall discharge or cause to be discharged any unpolluted waters as storm water, groundwater, roof runoff, subsurface drainage, or cooling water to any sanitary sewer except storm water runoff from limited areas, which storm water may be polluted at times, may be discharged to the sanitary sewer by permission of the Approving Authority.

SEC. 5-2-3
USER CONNECTIONS.


The owner(s) of all houses, buildings, or properties used for human occupancy, employment, recreation, or other purposes, situated within the City and abutting on any street, alley or right-of-way in which there is now located or may in the future be located a public sanitary sewer of the City is (are) hereby required at the owner(s) expense to install suitable toilet facilities therein, and connect such facilities directly to the proper public sewer in accordance with the provisions of this Chapter within three (3) months after the date of official notice from the Approving Authority to do so.

SEC. 5-2-4
STORM SEWERS.


Storm water other than that exempted under Section 5-2-2, and all other unpolluted drainage shall be discharged to such sewers as are specifically designated as combined sewers or storm sewers or to a natural outlet approved by the Approving Authority and other regulatory agencies.  Unpolluted industrial cooling water or process waters may be discharged, on approval of the Approving Authority, to a storm sewer or natural outlet.

SEC. 5-2-5
PROHIBITIONS AND LIMITATIONS ON SEWER DISCHARGE.

(a) Except as hereinafter provided, no person shall discharge or cause to be discharged any of the following described waters or wastes to any public sewer:

(1) Any gasoline, benzene, naphtha, fuel oil, other flammable or explosive liquid, solid, or gas.

(2) Any waters or wastes containing toxic or poisonous solids, liquids, or gases in sufficient quantity, either singly or by interaction with other wastes, to injure or interfere with any waste treatment process or which constitutes a hazard to humans or animals, creates a public nuisance, or creates any hazard in or has an adverse effect on the waters receiving any discharge from the treatment works.

(3) Any waters or wastes having a pH lower than 5.5 or higher than 9.0, or having any other corrosive property capable of causing damage or hazard to structures, equipment and personnel of the wastewater works.

(4) Solid or viscous substances in quantities or of such size capable of causing obstruction to the flow in sewers, or other interference with the proper operation of the wastewater facilities such as, but not limited to, ashes, cinders, sand, mud, straw, shavings, metal, glass, rags, feathers, tar, plastics, wood, unground garbage, whole blood, paunch manure, hair and fleshings, entrails, and paper dishes, cups, milk containers, etc., either whole or ground by garbage grinders.

(5) The following described substances, materials, waters, or waste shall be limited in discharges to municipal systems to concentrations or quantities which will not harm either the sewers wastewater treatment process or equipment, will not have an adverse effect on the receiving stream, or will not otherwise endanger lives, limb, public property, or constitute a nuisance.  The Approving Authority may set limitations lower than the limitations established in the regulations below if in his opinion such more severe limitations are necessary to meet the above objectives.  In forming his opinion as to the acceptability, the Approving Authority will give consideration to such factors as the quantity of subject waste in relation to flows and velocities in the sewers, materials of construction of the sewers, the wastewater treatment process employed, capacity of the wastewater treatment plant, and other pertinent factors.  The limitations or restrictions on materials or characteristics of waste or wastewaters discharged to the sanitary sewer which shall not be violated without approval of the Approving Authority are as follows:

a. Wastewater having a temperature higher than one hundred fifty (150°) degrees Fahrenheit (sixty-five [65°] degrees Celsius).

b. Wastewater containing more than twenty-five (25) milligrams per liter of petroleum oil, non-biodegradable cutting oils, or product of mineral oil origin.

c. Any water or waste which may contain more than 100 mg/1 of fate, oil or grease.

d. Any garbage that ha snot been properly shredded. Garbage grinders may be connected to sanitary sewers form homes, hotels, institutions, restaurants, hospitals, catering establishments, or similar places where garbage originates from the preparation of food in kitchens for the purpose of consumption on the premises or when served by caterers.

e. Any waters or wastes containing iron, chromium, copper, zinc, mercury, and similar objectionable or toxic substance to such degree that any such material received in the composite wastewater at the wastewater treatment works exceeds the limits established by the Approving Authority for such materials.

f. Any waters or wastes containing odor-producing substances exceeding limits which may be established by the Approving Authority.

g. Any radioactive wastes or isotopes of such half-life or concentration as may exceed limits established by the Approving Authority in compliance with applicable state or federal regulations.

h. Quantities of flow, concentrations, or both which constitute a “slug” as defined herein.

i. Water or wastes containing substances which are not amenable to treatment or reduction by the wastewater treatment processes employed, or are amenable to treatment only to such degree that the wastewater treatment plant effluent cannot meet the requirements of other agencies having jurisdiction over discharge to the receiving waters.

j. Any water or wastes which, by interaction with other water or wastes in the public sewer system, release obnoxious gases, form suspended solids which interfere with the collection system, or create a condition deleterious to structures and treatment processes.

k. Materials which exert or cause:

i. Unusual BOD, chemical oxygen demand or chlorine requirements in such quantities as to constitute a significant load on the wastewater treatment plant.

ii. Unusual volume of flow or concentration of wastes constituting “slugs” as defined herein.

iii. Unusual concentrations of inert suspended solids (such as, but not limited to, fuller’s earth, lime slurries, and lime residues) or of dissolved solids (such as, but not limited to, sodium sulfate).

iv. Excessive discoloration (such as, but not limited to dye wastes and vegetable tanning solutions).

(b) No wastewater, regardless of character, shall be discharge to the sewage system in such a manner as to interfere with the designed operation of the collection system or treatment facilities, or to cause the treatment works to exceed the limits presented by the WPDES permit.

SEC. 5-2-6
SPECIAL ARRANGEMENTS.


No Statement contained in this Article shall be construed as prohibiting any special agreement between the City and any person whereby an industrial waste of unusual strength or character may be admitted to the sewage disposal works, either before or after pretreatment, provided that there is not impairment of the functioning of the sewage disposal works by reason of the admission of such wastes, and no extra costs are incurred by the City without recompense by the person provided that all rates and provisions set forth in this Chapter are recognized and adhered to.

SEC. 5-2-7
NEW CONNECTIONS.


New connections to the sewage system shall not be permitted if there is insufficient capacity in the treatment works to adequately transport and/or treat, as required by the NPDES Permit, the additional wastewater anticipated to be received from such connections.

SEC. 5-2-8
ESTABLISHMENT OF SERVICE.


All newly installed service pipe shall be inspected and approved by the Crew Chief of Public Works prior to backfilling.

SEC. 5-2-9
RESERVED FOR FUTURE USE.

ARTICLE C

Control of High Strength – Toxic Wastes Directed to Public Sewers

SEC. 5-2-10
SUBMISSION OF BASIC DATA.

(a) Within three (3) months after passage of this Chapter, establishments discharging industrial wastes to a public sewer shall prepare and file with the Approving Authority a report that shall include pertinent data relating to the quantity and characteristics of the wastes discharged to the wastewater works.  Such information shall be provided as per Chapter NR 101 of the Wisconsin Administrative Code.

(b) Similarly, each establishment desiring to make a new connection to public sewer for the purpose of discharging industrial wastes shall prepare a file with the Approving Authority a report that shall include actual or predicted data relating to the quantity and characteristics of the waste to be discharged.

SEC. 5-2-11
EXTENSION OF TIME.


When it can be demonstrated that circumstances exist which would create an unreasonable burden on the establishment to comply with the time schedule imposed by Section 5-2-10, a request for extension of time may be presented for consideration of the Approving Authority.

SEC. 5-2-12
HIGH STRENGTH – TOXIC DISCHARGES.

(a) If any waters or wastes are discharged, or proposed to be discharged to the public sewers, which waters or wastes contain substance or possess the characteristics enumerated in Article B and which in the judgment of the Approving Authority, may have deleterious effect upon the sewage works, processes, equipment, or receiving waters, or which otherwise create a hazard to life, health, or constitute a public nuisance, the Approving Authority may:

1) Reject the wastes.

2) Require pretreatment to an acceptable limit for discharge to the public sewers.

3) Require control over the quantities and rates of discharge and/or

4) Require payment to cove the added cost of handling and treating the wastes not covered by existing taxes or sewer charges under the provisions of Section 205.

(b) The toxic pollutants subject to prohibition or regulation under this Article shall include, but need not be limited to, the list of toxic pollutants or combinations of pollutants established by Section 307(a) of the Clean Water Act of 1977 and subsequent amendments. Effluent standards or prohibitions for discharge to the sanitary sewer shall also conform to the requirements of Section 307(a) and associated regulations.

(c) Pretreatment standards for those pollutants which are determined not to be susceptible to treatment by the treatment works or which would interfere with the operation of such works shall conform to the requirements and associated regulations of Section 307(b) of the Clean Water Act of 1977 and subsequent amendments.  The primary source for such regulations shall be 40 CFR 403, General Pretreatment Regulations for Existing and New Sources of Pollution.

SEC. 5-2-13
CONTROL MANHOLES.

(a) Each user discharging industrial wastes into a public sewer may be required by the Approving Authority to construct and maintain one or more control manholes or access points to facilitate observation, measurement and sampling of his wastes, including domestic sewage.

(b) Control of manholes or access facilities shall be located and built in a manner acceptable to the Approving Authority.  If measuring devices are to be permanently installed they shall be of a type acceptable to the Approving Authority.

(c) Control manholes, access facilities, and relating equipment shall be installed by the establishment discharging the waste, at its expense, and shall be maintained by it so as to be in safe condition, accessible, and in proper operating condition at all times.  Plans for installation of the control manholes or access facilities and relating equipment shall be approved by the Approving Authority prior to the beginning of construction.

SEC. 5-2-14
METERING OF WASTE.


Devices for measuring volume of waste discharged may be required by the Approving Authority if this volume cannot otherwise be determined form the metered water consumption records.  Metering devices for determining the volume of waste shall be installed, owned, and maintained by the discharger.  A maintenance schedule must be accepted by the Approving Authority.  Following approval and installation such meters may not be removed without the consent of the Approving Authority.

SEC. 5-2-15
WASTE SAMPLING.

(a) Industrial wastes discharged into the public sewers shall be subject to periodic inspection and a determination of character and concentration of said wastes.  The determination shall be made by the industry as often as may be deemed necessary by the Approving Authority.

(b) Sampling shall be conducted in such a manner as to be representative of the composition of the wastes.  The sampling may be accomplished either manually or by the use of mechanical equipment acceptable to the Approving Authority.

(c) Installation, operation, and maintenance of the sampling facilities shall be the responsibility of the establishment discharging the waste and shall be subject to the approval of the Approving Authority.  Access to sampling locations shall be granted to the Approving Authority or its duly authorizing representative at all times.  Every care shall be exercised in the collection of samples to ensure their preservation in a state comparable to that at the time the sample was taken.

SEC. 5-2-16
ANALYSES.

(a) All measurements, tests, and analyses of the characteristics of waters and wastes to which reference is made in this ordinance shall be determined in accordance with the latest edition of “Standard Methods for the Examination of Water and Wastewater,” published by the American Public Health Association.  Sampling methods, location times, durations, and frequencies are to be determined on an individual basis subject to approval by the Approving Authority.

(b) Determination of the character and concentration of the industrial wastes shall be made by the establishment discharging them, or its agent, as designated and required by the Approving Authority.  The Approving Authority may also make its own analyses on the wastes and these determinations shall be binding as a basis for charges.

SEC. 5-2-17
PRETREATMENT.


Where required, in the opinion of the Approving Authority, to modify or eliminate wastes that are harmful to the structures, processes or operation of the wastewater treatment works, the person shall provide at his expense such preliminary treatment or processing facilities as may be determined required to render this wastes acceptable for admission to the public sewers.

SEC. 5-2-18
SUBMISSION OF INFORMATION.


Plans, specifications, any other pertinent information relating to proposed flow equalizations, pretreatment or processing facilities shall be submitted for review of the Approving Authority prior to the start of their construction if the effluent from such facilities is to be discharged into the public sewers.

SEC. 5-2-19
GREASE AND/OR SAND INTERCEPTORS.


Grease, oil, and sand interceptors shall be provided when, in the opinion of the Approving Authority, they are necessary for the proper handing of liquid wastes containing floatable grease in excessive amounts, as specified in Section 5-2-5(a)(5)c, or any flammable wastes, sand, or other harmful ingredients; except that such interceptors shall not be required for private living quarters or dwelling units.  All interceptors shall be of a type and capacity approved by the Approving Authority, and shall be located as to be readily and easily accessible for cleaning and inspection.  In the maintaining of these interceptors of the owner(s) shall be responsible for the proper removal and disposal by appropriate means of the captured material and shall maintain records of the dates, and means of disposal which are subject tot review by the Approving Authority.  Any removal and hauling of the collected materials not performed by owner(s) personnel must be performed by currently licensed waste disposal firms.

ARTICLE D

Right of Entry, Safety and Identification

SEC. 5-2-20
RIGHT OF ENTRY.


The superintendent of Water Works, Plumbing Inspector, or other duly authorized employee of the City of Loyal bearing proper credentials and identification shall be permitted to enter all properties for the purpose of inspection, observation, testing, all in accordance with the provisions of this Chapter and State laws.  The Superintendent of Water Works, Plumbing Inspector, or other duly authorized employee of the City of Loyal shall have no authority to inquire into any process beyond that point having a direct bearing on the kind and source of discharge to the sewers or waterways or facilities for wastewater treatment.

SEC. 5-2-21
SAFETY.


While performing the necessary work on private premises referred to in Section 5-2-20, the duly authorized City employees shall observe all safety rules applicable to the premises established by the owner or occupant and the City shall indemnify the owner against loss or damage to its property by City employees and against the liability claims and demands for personal injury or property damage asserted against the owner and growing out of gauging and sampling operation of City employees; except as such may be caused by negligence or failure of the owner to maintain safe conditions as required in Section 5-2-13.

SEC. 5-2-22
IDENTIFICATION, RIGHT TO ENTER EASEMENTS.


The Superintendent of Water Works and other duly authorized employees of the City of Loyal bearing proper credentials and identification shall be permitted to enter all private properties through which the City holds a duly negotiated easement for the purpose of, but not limited to, inspection, observation, measurement, sampling, repair and maintenance of any portion of the sewage works lying within said easement, all subject to the terms, if any, of the agreement.

SEC. 5-2-23 THROUGH SEC. 5-2-29
RESERVED FOR FUTURE USE.

ARTICLE E

Sewer Construction (Building Sewers)

SEC. 5-2-30
WORK AUTHORIZED.

a) No unauthorized person shall uncover, make any connections with or opening into, use, alter, or disturb any public sewer or appurtenance thereof without first obtaining a written permit from the Approving Authority.  The fee for this permit shall be Five ($5.00) Dollars, and shall be paid prior to the issuance of the permit.

b) No contractor, plumber, pipe fitter or other person shall be permitted to work on any public sewer or building sewer without first receiving a license from the State of Wisconsin, except in cases where State law permits building owners to do their own work without being licensed.

c) Prior to the commencement of the work, the permittee shall notify the Approving Authority at least forty-eight (48) hours before beginning excavations.

SEC. 5-2-31
COST OF SEWER CONNECTION.


All costs and expenses incident to the installation and connection o the building sewer shall be borne by the owner.  The owner shall indemnify the City from any loss or damage that may directly or indirectly be occasioned by the installation of the building sewer.

SEC. 5-2-32
USE OF OLD BUILDING SEWERS.


Old building sewers may be used in connection with new buildings only when they are found on examination and test by the Approving Authority, to meet all requirements for this Chapter.

SEC. 5-2-23
MATERIALS AND METHODS OF CONSTRUCTION.


The size, slope, alignment, materials of construction of building sewer, and the methods to be used in excavating, placing of the pipe, jointing, testing, and backfilling the trench, shall all conform to the requirements of the Wisconsin Administrative Code, the State Department of Natural Resources, the building and plumbing code, and other applicable rules and regulation of the City of Loyal.  In the absence of code provisions or an amplification thereof, the materials and procedure set forth in appropriate specifications of the A.S.T.M. and W.P.C.F. Manual of Practice No. 9 shall apply.

SEC. 5-2-34
BUILDING SEWER GRADE.


Whenever possible, the building sewer shall be brought to the building at an elevation below the basement floor.  In all buildings in which any building drain is too low to permit gravity flow to the public sewer, sanitary sewage carried by such building drain shall be lifted by an approved means and discharged to the building sewer.

SEC. 5-2-35
STORM AND GROUNDWATER DRAINS.

(a) No person shall make connection of roof downspouts, exterior foundation drains, areaway drains, or other sources of surface runoff or groundwater to a building sewer or building drain which is connected directly or indirectly to a public sanitary sewer.

(b) All existing downspouts or groundwater drains, etc. connect directly or indirectly to a public sanitary sewer must be disconnected no later than sixty (60) days form the date of an official written notice by the Approving Authority.  Exceptions to the above shall be made only by the Approving Authority in writing.

SEC. 5-2-36
CONFORMANCE TO PLUMBING CODES.

(a) The connection of the building sewer into the public sewer shall conform to the requirement of the building and plumbing code or other applicable rules and regulations of the City of Loyal or the procedures set forth in appropriate specifications of the A.S.T.M. and the W.P.C.F. Manual of Practice No. 9.  All such connections shall be made gastight and watertight.  Any deviation from the prescribed procedures and materials must be approved by the Approving Authority before installation.

(b) All connections to existing sewer mains shall be made with a saddle “T” or “Y” fitting set up on a carefully cut opening centered in the upper quadrant of the main sewer pipe and securely strapped on with corrosion-resistant straps or rods, or with solvent-welded joints in the case of plastic pipe.

SEC. 5-2-37
INSPECTION OF CONNECTION.


The applicant for the building sewer permit shall notify the Approving Authority when the building sewer is ready for inspection and connection to the public sewer.  The connection shall be made under the supervision of the Approving Authority.

SEC. 5-2-38
BARRICADES; RESTORATION.


All excavations for the building sewer installation shall be adequately guarded with barricades and lights so as to protect the public from hazard.  Streets, sidewalks, parkways and other public property disturbed in the course of work shall be restored in a manner satisfactory to the City.

SEC. 5-2-39
RESERVED FOR FUTURE USE.

ARTICLE F

Septic Tank and Holding Tank Disposal

SEC. 5-2-40
DISCHARGE OF HOLDING TANK WASTES INTO SYSTEM.


No person in the business of gathering and disposing of septic tank sludge or holding tank  sewage shall transfer such material into any portion of the City of Loyal treatment works.

SEC. 5-2-41 THROUGH SEC. 5-2-49
RESERVED FOR FUTURE USE.

ARTICLE G

Damage or Tampering With Sewage Facilities

SEC. 5-2-50
WILLFUL, NEGLIGENT OR MALICIOUS DAMAGE.


No unauthorized person shall maliciously, willfully or negligently break, damage, destroy, uncover, deface, or tamper with any structure, appurtenance, or equipment which is a part of the sewage facilities.  Any persons violating this provision shall be subject to immediate arrest under a charge of disorderly conduct.

SEC. 5-2-51
LIABILITY TO DISCHARGEER FOR LOSSES.


Any person who intentionally, negligently or accidentally violates any provisions of this Chapter shall become liable to the City or any downstream user, for any expense, loss or damage occasioned by reason of such violation which the City or any downstream user may suffer as a result thereof.  This section shall be applicable whether or not a written notice of the violation was given as provided in Section 5-2-60 and without consideration for any penalties which may be imposed for a violation of this Chapter.

SEC. 5-2-52 THROUGH SEC. 5-2-59
RESERVED FOR FUTURE USE.

ARTICLE H

Violations and Penalties

SEC. 5-2-60
WRITTEN NOTICE OF VIOLATION.


Any person found to be violating any provision of this Ordinance shall be served by the City with a written notice stating the nature of the violation and providing a reasonable time for the satisfactory correction thereof.  The offender shall, within the period of time stated in such notice, permanently cease all violations.

SEC. 5-2-61
ACCIDENTIAL DISCHARGE.


Any person found to be responsible for accidentally allowing a deleterious discharge into the sewer system which causes damage to the treatment facility and/or receiving body of water shall, in addition to a fine, pay the amount of cover damage, both values to be established by the Approving Authority.

SEC. 5-2-62
CONTINUED VIOLATIONS.


Any person, partnership or corporation, or any officer, agent or employee thereof, who shall continue any violation beyond the aforesaid notice time limit provided shall, upon conviction thereof, forfeit not less than One Hundred ($100.00) Dollars nor more than Five Hundred ($500.00) Dollars, together with the cost of prosecution.  In default of payment of such forfeiture and costs, said violator shall be imprisoned in the Clark County Jail for a period not to exceed thirty (30) days.  Each day in which any violation is continued beyond the aforesaid notice time limit shall be deemed a separate offense.

SEC. 5-2-63
LIABILITY TO THE CITY FOR LOSSES.


Any person violating any provision of this Chapter shall become liable to the City for any expense, loss, or damage occasioned by reason of such violation which the City may suffer as a result thereof.  

SEC. 5-2-64 THROUGH SEC. 5-2-69
RESERVED FOR FUTURE USE.

ARTICLE I

Appeals

SEC. 5-2-70
PROCEDURES.

(a) Any user, permit applicant, or permit holder affected by any decision, action, or determination, including cease and desist orders, made by the Approving Authority interpreting or implementing the provisions of this Chapter or in any permit issued herein, may file with the Approving Authority a written request for reconsideration with ten (10) days setting forth in detail the facts supporting the user’s request for reconsideration.  The Approving Authority shall render a decision on the request for reconsideration to the user, permit applicant or permit holder in writing within fifteen (15) days of receipt of request.  If the ruling on the request for reconsideration made by the Approving Authority is unsatisfactory, the person requesting reconsideration may, within ten (10) days after notification of the Approving Authority’s action, file a written appeal with the City Clerk.

(b) A fee of One Hundred ($100.00) Dollars shall accompany any appeal for a ruling by the City Council.  This fee may be refunded if the appeal is sustained in favor of the appellant.

(c) The written appeal shall be heard by the City Council with forty-five (45) days from the date of filing.  The City Council shall make a final ruling on the appeal within sixty (60) days from the date of filing. 

SEC. 5-2-71 THROUGH SEC. 5-2-79
RESERVED FOR FUTURE USE.

ARTICLE J

Validity

SEC. 5-2-80
REPEAL OF CONFLICTING ORDINANCES.


All ordinances or parts of ordinances or regulations or part of regulations in conflict with this Chapter are hereby repealed.

SEC. 5-2-81
INVALIDATION CLAUSE.


Invalidity of any section, clause, sentence, or provision in the Chapter shall not affect the validity of any other section, clause, sentence, provision or this Chapter which can be given effect without such invalid part or parts.

SEC. 5-2-82
AMENDMENT.


The City of Loyal through its duly qualified officers, reserves the right to amend this Chapter in part or in whole wherever it may deem necessary, but such right will be exercised only after due notice to all persons concerned and proper hearing on the proposed amendment.

SEC. 5-2-83 THROUGH SEC. 5-2-89
RESERVED FOR FUTURE USE.

ARTICLE K

Audit

SEC. 5-2-20
ANNUAL AUDIT.


The City of Loyal shall conduct an annual audit, the purpose of which shall be to demonstrate the continued proportionality and sufficiency of the user charges relative to changes in system operation and maintenance costs.

SEC. 5-2-91 THROUGH SEC. 5-2-99
RESERVED FOR FUTURE USE.

ARTICLE L

Enacting Clause

SEC. 5-2-100
DATE OF EFFECT.


This Chapter shall take effect and be in force on and after January 1, 1984.

